SuperLead

2D Barcode Reader User Guide B

V1.7

Table of Contents

Disclaimer	4

1	Getting Started	5

1.1	About This Guide	5

1.2	Barcode Scanning	5

1.3	Factory Defaults	5

2	Communication Interfaces	6

2.1	TTL-232 Interface	6

2.2	Baud Rate	7

2.3	Data Bit & Parity Check& Stop Bit	8

2.4	USB HID-KB	8

2.5	USB Country Keyboard Types	8

2.6	Convert Case	9

2.7	USB COM Port Emulation	9

3	General Configuration	10

3.1	Trigger Mode	10

3.2	Presentation Mode	10

3.3	Cell Phone Mode	11

3.4	Inverse color	12

3.5	Illumination for Trigger Mode	12

3.6	Illumination for Presentation Mode	12

3.7	Beeper - Good Read	14

3.8	Beeper Tone - Good Read	14

3.9	Beeper Duration - Good Read	14

3.10	Beeper Number - Good Read	14

3.11	Barcode Scanning Delay	15

4	Data Formatting	16

4.1	General Configuration	16

4.2	Add Prefix	17

4.3	Add Suffix	18

4.4	Clear All Prefix and Suffix	18

5	Symbologies	19

5.1	General Settings	19

5.2	1D Symbologies	20

5.2.1	Code 128	20

5.2.2	EAN-8	21

5.2.3	EAN-13	23

5.2.4	UPC-E	25

5.2.5	UPC-A	28

5.2.6	Interleaved 2 of 5	30

5.2.7	Matrix 2 of 5	31

5.2.8	Industrial 2 of 5	32

5.2.9	Code 39	33

5.2.10	Coda bar	35

5.2.11	Code 93	37

5.2.12	GS1-128	38

5.2.13	MSI	39

5.3	2D Symbologies	41

5.3.1	PDF 417	41

5.3.2	Micro PDF 417	42

5.3.3	QR Code	43

5.3.4	Data Matrix	44

5.3.5	Maxi code	45

5.3.6	Aztec	46

5.3.7	Hanxin	47

5.4	Postal Symbologies	48

5.4.1	China Postal Code	48

5.4.2	Telepen	49

6	Serial Commands	50

6.1	Function Commands	50

6.2	Menu Commands	50

6.3	Menu number	51

7	Appendix	59

7.1	Appendix 1: AIM ID Table	59

7.2	Appendix 2: ASCII Table	61

7.3	Appendix 3: Digit Barcodes	65

# Disclaimer

©2013 Suzhou SuperLead Smart System Co., Ltd (abbreviated as SUPERLEAD). All rights reserved.

Please read through the manual carefully before using the product and operate it according to the manual. It is advised that you should keep this manual for future reference.

Do not disassemble the device or remove the seal label from the device. Otherwise, Suzhou SuperLead Smart System Co., Ltd. does not assume responsibility for the warranty or replacement.

All pictures in this manual are for reference only and actual product may differ. To improve reliability, function, or design, SUPERLEAD reserves the right to make changes to any software or product at any time without notice. Besides, the information contained herein is subject to change without prior notice.

The products depicted in this manual may include software copyrighted by SUPERLEAD or a third party. The user, corporation or individual, shall not duplicate, in whole or in part, distribute, modify, decompile, disassemble, decode, reverse engineer, rent, transfer or sublicense such software without prior written consent from the copyright holders.

This manual is copyrighted. No part of this publication may be reproduced or used in any form without written permission from SUPERLEAD.

SUPERLEAD reserves the right to make final interpretation of the statement above Smart System Co., Ltd.

http://www.isupermax.com

# Getting Started

## About This Guide

This guide provides programming instructions for the 2D BARCODER READER. Users can configure the 2D BARCODER READER by scanning the programming barcodes included in this manual.

## Barcode Scanning

Powered by area-imaging technology and 2D BARCODER READER patented “Hercules” technology, the 2D BARCODER READER features fast scanning and decoding accuracy. Barcodes rotated at any angle can still be read with ease. When scanning a barcode, simply center the aiming dot/beam or pattern projected by the 2D BARCODER READER over the barcode.

## Factory Defaults

Scanning the following barcode can restore the engine to the factory defaults.

Note: Use this feature with discretion.

Restore All Factory Defaults

# Communication Interfaces

The 2D BARCODER READER provides a TTL-232 interface and a USB interface to communicate with the host device. The host device can receive scanned data and send commands to control the engine or to access/alter the configuration information of the engine via the TTL-232 or USB interface.

## TTL-232 Interface

Before using Serial Communication interface, scanner must be configured as 232 interface.

232 Interface

Serial communication interface is usually used when connecting the engine to a host device (like PC, POS). However, to ensure smooth communication and accuracy of data, you need to set communication parameters (including baud rate, parity check, data bit and stop bit) to match the host device.

The serial communication interface provided by the engine is based on TTL-level signals. TTL-232 can be used for most application architectures. For those requiring RS-232, an external conversion circuit is needed. The conversion circuit is available only to some models.

Default serial communication parameters are listed below. Make sure all parameters match the host requirements.

| Parameter | Factory Default |
| --- | --- |
| Serial Communication | Standard TTL-232 |
| Baud Rate | 115200 |
| Parity Check | None |
| Data Bits | 8 |
| Stop Bits | 1 |
| Hardware Flow Control | None |

## Baud Rate

Baud rate is the number of bits of data transmitted per second. Set the baud rate to match the Host requirements.

| 2400 | 4800 |
| --- | --- |
| 9600 | 19200 |
| 38400 | 57600 |
| 115200 (default) |  |

## Data Bit & Parity Check& Stop Bit

| None Parity /8 Data Bits/1 Stop Bit(default) | Even Parity /8 Data Bits/1 Stop Bit |
| --- | --- |
| Odd Parity /8 Data Bits/1 Stop Bit |  |

## USB HID-KB

When you connect the engine to the Host via a USB connection, you can enable the USB HID-KB feature by scanning the barcode below. Then engine’s transmission will be simulated as USB keyboard input. The Host receives keystrokes on the virtual keyboard. It works on a Plug and Play basis and no driver is required.

USB HID-KB

## USB Country Keyboard Types

Keyboard layouts vary from country to country. The default setting is 1-U.S. keyboard.

| 1 – U.S. (Default) |  |  |
| --- | --- | --- |
|  | 2 – UK |  |
| 3 – Denmark |  |  |
|  | 4 – France |  |
| 5 – Finland |  |  |
|  | 6 – Turkey_F |  |
| 7 – Italy |  |  |
|  | 8 – Norway |  |
| 9 – Albania |  |  |
|  | 10 – Belgium |  |
| 11 – Bosnia |  |  |
|  | 12 – Brazil |  |
| 13 – Croatia |  |  |
|  | 14 – Czech |  |
| 15 – Dutch |  |  |
|  | 16 – Estonia |  |
| 17 – Germany |  |  |
|  | 18 – Greek |  |
| 19 – Hungary |  |  |
|  | 20 – Irish |  |
| 21 – Latvia |  |  |
|  | 22 – Lithuania |  |
| 23 – Macedonia |  |  |
|  | 24 – Spain |  |
| 25 – Poland |  |  |
|  | 26 – Portugal |  |
| 27 – Romania |  |  |
|  | 28 – Russia |  |
| 29 – Japan |  |  |

## Convert Case

Scan the appropriate barcode below to convert barcode data to your desired case.

| No Case Conversion (default) |  |
| --- | --- |
| Convert All to Upper Case | Convert All to Lower Case |

Example: When the Convert All to Lower Case feature is enabled, barcode data “AbC” is transmitted as “abc”.

## USB COM Port Emulation

If you connect the engine to the Host via a USB connection, the USB COM Port Emulation feature allows the Host to receive data in the way as a serial port does. A driver is required for this feature.

USB COM Port Emulation

# General Configuration

## Trigger Mode

If the Trigger Mode is enabled, you could activate the scanner by providing an external hardware trigger, or using a serial trigger command. When in manual trigger mode, the scanner scans until a barcode is read, or until the hardware trigger is released. When in serial mode, the scanner scans until a barcode has been read or until the deactivate command is sent.

Trigger Mode(default)

## Presentation Mode

This set the scanner to work in Presentation mode.

Presentation Mode

## Cell Phone Mode

If the Cell Phone Mode is enabled, the engine activates a special capturing image and illumination session.

If you want to enable Cell Phone reading in Trigger Mode, please scan below barcode:

Trigger Mode with Cell Phone reading

If you want to enable Cell Phone reading in Presentation Mode, please scan below barcode:

Presentation Mode with Cell Phone reading

## Inverse color

| Off(default) | Only Inverse On | Inverse and Normal Both On |
| --- | --- | --- |

## Illumination for Trigger Mode

| Illumination setting for Trigger Mode |  |
| --- | --- |
| High Level Illumination | Normal Level Illumination 1(default) |
| Mid Level Illumination | Low Level Illumination |
| Illumination OFF |  |

## Illumination for Presentation Mode

| Illumination setting when in Scanning |  |
| --- | --- |
| High Level Illumination | Normal Level Illumination(default) |
| Mid Level Illumination | Low Level Illumination |
| Illumination OFF |  |
| Illumination setting when in Idle |  |
| High Level Illumination | Normal Level Illumination |
| Mid Level Illumination | Low Level Illumination(default) |
| Illumination OFF |  |

## Beeper - Good Read

| On (default) | Off |
| --- | --- |

## Beeper Tone - Good Read

| Low | Middle | High (default) |
| --- | --- | --- |

## Beeper Duration - Good Read

| Normal (default) | Short |
| --- | --- |

## Beeper Number - Good Read

| 1 (default) | 2 |
| --- | --- |
| 3 | 4 |
| 5 | 6 |

## Barcode Scanning Delay

| No Delay (default) | Delay 500MS |
| --- | --- |
| Delay 2000ms |  |

# Data Formatting

## General Configuration

| Add CR | Add LF |
| --- | --- |
| Add CRLF | Add TAB |

## Add Prefix

| Set Custom Prefix |  |
| --- | --- |
| Save | Not Save |

To set a customer prefix, scan the “Set Custom Prefix” barcode and the numeric barcodes which representing the hexadecimal values of a desired prefix, and then scan the “Save” barcode. Refer to Appendix 2: ASCII Table for hexadecimal values of characters.

Example: Set the custom Prefix to “ODE”

1.	Check the hex values of “ODE” in the ASCII Table. (“ODE”: 4F, 44, 45)

2.	Scan the Set Custom Prefix barcode.

3.	Scan the numeric barcodes”9”,”9”, “4”, “F”, “4”, “4”, “4” and “5”in Appendix 5 .

4.	Scan the Save barcode.

## Add Suffix

| Set Custom Suffix |  |
| --- | --- |
| Save | Not Save |

To set a customer suffix, scan the “Set Custom Suffix” barcode and the numeric barcodes which representing the hexadecimal values of a desired suffix, and then scan the “Save” barcode. Refer to Appendix 4: ASCII Table for hexadecimal values of characters.

Example: Set the custom Suffix to “ODE”

1.	Check the hex values of “ODE” in the ASCII Table. (“ODE”: 4F, 44, 45)

2.	Scan the Set Custom Suffix barcode.

3.	Scan the numeric barcodes”9”,”9”, “4”, “F”, “4”, “4”, “4” and “5”in Appendix 5 .

4.	Scan the Save barcode.

## Clear All Prefix and Suffix

Clear all prefix and suffix (default)

# Symbologies

## General Settings

Enable/Disable All Symbologies

If  the  Disable  All  Symbologies  feature  is  enabled,  the  engine  will  not  be  able  to  read  any non-programming barcodes except the programming barcodes.

| Enable All Symbologies | Disable All Symbologies |
| --- | --- |

## 1D Symbologies

## Code 128

Restore Factory Defaults

Restore the Factory Defaults of Code 128 (default)

Enable/Disable Code 128

| Enable Code 128 (default) | Disable Code 128 |
| --- | --- |

Message Length

Message length can be set to the maximum value or minimum value. The value between the maximum and the minimum is valid.

The maximum value and minimum value can be set using “Programming Command”. Please check the programming command guide for the detail.

Code 128 max length command: 020A03. The parameter of this command can be set from min to 90.

Code 128 min length command: 020A02. The parameter of this command can be set from 0 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command:

Max: 020A0325.

Min: 020A0210.

## EAN-8

Restore Factory Defaults

Restore the Factory Defaults of EAN-8 (default)

Enable/Disable EAN-8

| Enable EAN-8 (default) | Disable EAN-8 |
| --- | --- |

Transmit Check Digit

EAN-8 is 8 digits in length with the last one as its check digit used to verify the accuracy of the data.

| Transmit EAN-8 Check Digit (default) | Do Not Transmit EAN-8 Check Digit |
| --- | --- |

Add-On Code

An EAN-8 barcode can be augmented with a two-digit or five-digit add-on code on code to form a new one. In the examples below, the part surrounded by blue dotted line is an EAN-8 barcode while the part circled by red dotted line is add-on code.

| Enable 2-Digit Add-On Code | Disable 2-Digit Add-On Code (default) |
| --- | --- |
| Enable 5-Digit Add-On Code | Disable 5-Digit Add-On Code (default) |

Add-On Code Required

| EAN-8 Add-On Code Required | EAN-8 Add-On Code Not Required (default) |
| --- | --- |

ENA/JAN-8 Addenda Separator

When this feature is enabled, there is a space between barcode and addenda. When this feature is disabled, there is no space.

| Enable EAN/JAN-8 Addenda Separator (default) | Disable EAN/JAN-8 Addenda Separator |
| --- | --- |

## EAN-13

Restore Factory Defaults

Restore the Factory Defaults of EAN-13 (default)

Enable/Disable EAN-13

| Enable EAN-13 (default) | Disable EAN-13 |
| --- | --- |

Transmit Check Digit

| Transmit EAN-13 Check Digit (default) | Do Not Transmit EAN-13 Check Digit |
| --- | --- |

Add-On Code

| Enable 2-Digit Add-On Code | Disable 2-Digit Add-On Code (default) |
| --- | --- |
| Enable 5-Digit Add-On Code | Disable 5-Digit Add-On Code (default) |

Add-On Code Required

| EAN-13 Add-On Code Required | EAN-13 Add-On Code Not Required (default) |
| --- | --- |

ENA/JAN-13 Addenda Separator

When this feature is enabled, there is a space between barcode and addenda. When this feature is disabled, there is no space.

| Enable ENA/JAN-13 Addenda Separator (default) | Disable ENA/JAN-13 Addenda Separator |
| --- | --- |

ISBN Translate

When enable this feature and is scanned, ENA13 Book land symbols are translated into their equivalent ISBN number format.

| Enable ISBN Translate | Disable ISBN Translate (default) |
| --- | --- |

## UPC-E

Restore Factory Defaults

Restore the Factory Defaults of UPC-E (default)

Enable/Disable UPC-E0/E1

| Enable UPC-E0 (default) | Disable UPC-E0 |
| --- | --- |
| Enable UPC-E1(default) | Disable UPC-E1 |

UPCE0 Check Digit

| Enable UPC-E0 Check Digit (default) | Disable UPC-E0 Check Digit |
| --- | --- |

UPCE0 Expand

UPCE0 expand expands the UPCE code to the 12 digits, UPC-A format.

| Enable UPC-E0 Expand | Disable UPC-E0 Expand (default) |
| --- | --- |

UPCE0 Addenda Required

When required is scanned, the scanner will only read UPC-E barcodes that have addenda.

| Enable UPC-E0 Required | Disable UPC-E0 Required (default) |
| --- | --- |

UPCE0 Addenda Separator

| Enable UPC-E0 Separator (default) | Disable UPC-E0 Separator |
| --- | --- |

UPCE0 Number System

The number system digit of UPC symbol is normally transmitted at the beginning of the scanned data, but the unit can be programmed so it will be not transmitted.

| Enable UPC-E0 Number System (default) | Disable UPC-E0 Number System |
| --- | --- |

UPCE0 Addenda

| Enable 2 Digit Addenda | Disable 2 Digit Addenda (default) |
| --- | --- |
| Enable 5 Digit Addenda | Disable 5 Digit Addenda (default) |

## UPC-A

Restore Factory Defaults

Restore the Factory Defaults of UPC-A (default)

Enable/Disable UPC-A

| Enable UPC-A (default) | Disable UPC-A |
| --- | --- |

UPC-A Check Digit

| Enable UPC-A Check Digit (default) | Disable UPC-A Check Digit |
| --- | --- |

UPC-A Addenda Required

When required is scanned, the scanner will only read UPC-E barcodes that have addenda.

| Enable UPC-A Required | Disable UPC-A Required (default) |
| --- | --- |

UPC-A Addenda Separator

| Enable UPC-A Separator (default) | Disable UPC-A Separator |
| --- | --- |

UPC-A: Number System

The number system digit of UPC symbol is normally transmitted at the beginning of the scanned data, but the unit can be programmed so it will be not transmitted.

| Enable UPC-A Number System (default) | Disable UPC-A Number System |
| --- | --- |

UPC-A: Addenda

| Enable 2 Digit Addenda | Disable 2 Digit Addenda (default) |
| --- | --- |
| Enable 5 Digit Addenda | Disable 5 Digit (default) |

## Interleaved 2 of 5

Restore Factory Defaults

Restore the Factory Defaults of Interleaved 2 of 5 (default)

Enable/Disable Interleaved 2 of 5

| Enable Interleaved 2 of 5 (default) | Disable Interleaved 2 of 5 |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The data between the maximum and the minimum is valid.

The maximum value and minimum value can be set using Programming Command. Please check the programming command guide for the detail.

Interleaved 2 of 5 max length command: 020404. The parameter of this command can be set from min to 80.

Interleaved 2 of 5 min length command: 020403. The parameter of this command can be set from 2 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 02040425.

Min: 02040310.

Interleaved 2 of 5 Check Digit

| No check Char (default) | Validate and Transmit | Validate not Transmit |
| --- | --- | --- |

## Matrix 2 of 5

Restore Factory Defaults

Restore the Factory Defaults of Matrix 2 of 5 (default)

Enable/Disable Matrix 2 of 5

| Enable Matrix 2 of 5 | Disable Matrix 2 of 5 (default) |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The value is valid between the maximum and the minimum.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

Matrix 2 of 5 max length command: 020803. The parameter of this command can be set from min to 80.

Matrix 2 of 5 min length command: 020802. The parameter of this command can be set from 1 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 02080325.

Min: 02080210.

## Industrial 2 of 5

Restore Factory Defaults

Restore the Factory Defaults of Industrial 2 of 5 (default)

Enable/Disable Industrial 2 of 5

| Enable Industrial 2 of 5 | Disable Industrial 2 of 5 (default) |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The value is valid between the maximum and the minimum.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

Industrial 2 of 5 max length command: 020603. The parameter of this command can be set from min to 48.

Industrial 2 of 5 min length command: 020602. The parameter of this command can be set from 1 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 02060325.

Min: 02060210.

## Code 39

Restore Factory Defaults

Restore the Factory Defaults of Code 39 (default)

Enable/Disable Code 39

| Enable Code 39 (default) | Disable Code 39 |
| --- | --- |

Transmit Start/Stop Character

| Transmit Start/Stop Character | Do not Transmit Start/Stop Character (default) |
| --- | --- |

Code 39 Check Character

| No Check Char (default) | Validate and Transmit | Validate no Transmit |
| --- | --- | --- |

Code 39 Full ASCII

| Enable Code 39 Full ASCII | Disable Code 39 Full ASCII (default) |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The value is valid between the maximum and the minimum.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

Code 39 max length command: 020308. The parameter of this command can be set from min to 48.

Code 39 min length command: 020307. The parameter of this command can be set from 0 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 02030825.

Min: 02030710.

## Coda bar

Restore Factory Defaults

Restore the Factory Defaults of Coda bar (default)

Enable/Disable Codabar

| Enable Coda bar (default) | Disable Coda bar |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The data between the maximum and the minimum is valid.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

Coda bar max length command: 020206. The parameter of this command can be set from min to 60.

Coda bar min length command: 020205. The parameter of this command can be set from 2 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 02020625.

Min: 02020510.

Transmit Start/Stop Character

| Transmit Start/Stop Character | Do not Transmit Start/Stop Character (default) |
| --- | --- |

Coda bar Check Character

| No Check Char (default) | Validate and Transmit |
| --- | --- |
| Validate no Transmit |  |

## Code 93

Restore Factory Defaults

Restore the Factory Defaults of Code 93 (default)

Enable/Disable Code 93

| Enable Code 93 | Disable Code 93 (default) |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The data between the maximum and the minimum is valid.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

Code 93 max length command: 020D03. The parameter of this command can be set from min to 80.

Code 93 min length command: 020D02. The parameter of this command can be set from 0 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 020D0325.

Min: 020D0210.

## GS1-128

Restore Factory Defaults

Restore the Factory Defaults of GS1-128 (default)

Enable/Disable GS1-128

| Enable GS1-128 (default) | Disable GS1-128 |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The data between the maximum and the minimum is valid.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

GS1-128 max length command: 020B03. The parameter of this command can be set from min to 80.

GS1-128 min length command: 020B02. The parameter of this command can be set from 0 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 020B0325.

Min: 020B0210.

## MSI

Restore Factory Defaults

Restore the Factory Defaults of MSI (default)

Enable/Disable MSI

| Enable MSI | Disable MSI (default) |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The data is valid between the maximum and the minimum.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

MSI max length command: 020E04. The parameter of this command can be set from min to 48.

MSI min length command: 020E03. The parameter of this command can be set from 4 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 020E0425.

Min: 020E0310.

MSI Check Character

| Validate Type 10, No Transmit (Default) |  |
| --- | --- |
| Validate Type 10, Transmit | Validate 2 Type 10 Characters, No Transmit |
| Validate 2 Type 10 Characters, Transmit | Validate Type 11 Then Type 10 Character, No Transmit |
| Validate Type 11 Then Type 10 Characters, Transmit | Disable MSI Check Characters |

## 2D Symbologies

## PDF 417

Restore Factory Defaults

Restore the Factory Defaults of PDF 417 (default)

Enable/Disable PDF 417

| Enable PDF 417 (default) | Disable PDF 417 |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The data is valid between the maximum and the minimum.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

PDF417 max length command: 021F06. The parameter of this command can be set from min to 2750.

PDF417 min length command: 021F05. The parameter of this command can be set from 1 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 021F0625.

Min: 021F0510.

## Micro PDF 417

Restore Factory Defaults

Restore the Factory Defaults of Micro PDF 417 (default)

Enable/Disable Micro PDF 417

| Enable PDF 417 | Disable PDF 417 (default) |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The data is valid between the maximum and the minimum.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

PDF417 max length command: 022003. The parameter of this command can be set from min to 2750.

PDF417 min length command: 022002. The parameter of this command can be set from 1 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 02200325.

Min: 02200310.

## QR Code

Restore Factory Defaults

Restore the Factory Defaults of QR Code (default)

Enable/Disable QR Code

| Enable QR Code (default) | Disable QR Code |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The data is valid between the maximum and the minimum is valid.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

QR max length command: 023703. The parameter of this command can be set from min to 7089.

QR min length command: 023702. The parameter of this command can be set from 1 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 02370325.

Min: 02370210.

## Data Matrix

Restore Factory Defaults

Restore the Factory Defaults of Data Matrix (default)

Enable/Disable Data Matrix

| Enable Data Matrix (default) | Disable Data Matrix |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The data is valid between the maximum and the minimum.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

Data Matrix max length command: 023603. The parameter of this command can be set from min to 3116.

Data Matrix min length command: 023602. The parameter of this command can be set from 1 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 02360325.

Min: 02360210.

## Maxi code

Restore Factory Defaults

Restore the Factory Defaults of Maxi code (default)

Enable/Disable Maxi code

| Enable Maxi code | Disable Maxi code (default) |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The data is valid between the maximum and the minimum.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

Maxi Code max length command: 023403. The parameter of this command can be set from min to 150.

Maxi Code min length command: 023402. The parameter of this command can be set from 1 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 02340325.

Min: 02340210.

## Aztec

Restore Factory Defaults

Restore the Factory Defaults of Aztec (default)

Enable/Disable Aztec

| Enable Aztec | Disable Aztec (default) |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The data is valid between the maximum and the minimum.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

Aztec max length command: 023306. The parameter of this command can be set from min to 3832.

Aztec min length command: 023305. The parameter of this command can be set from 1 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 02330625.

Min: 02330510.

## Hanxin

Restore Factory Defaults

Restore the Factory Defaults of Hanxin (default)

Enable/Disable Hanxin

| Enable Hanxin | Disable Hanxin (default) |
| --- | --- |

Message Length

Message length can be set to the maximum value, minimum value. The data is valid between the maximum and the minimum.

The maximum value and minimum value can be set using Programming command. Please check the programming command guide for the detail.

Hanxin max length command: 023803. The parameter of this command can be set from min to 7833.

Hanxin min length command: 023802. The parameter of this command can be set from 1 to max.

Example: Set the Barcode Message length of the minimum value is 10; the maximum value is 25.

Programming command

Max: 02380325.

Min: 02380210

## Postal Symbologies

## China Postal Code

Restore Factory Defaults

Restore the Factory Defaults of China Postal Code (default)

Enable/Disable China Postal Code

| Enable China Postal Code | Disable China Postal Code (default) |
| --- | --- |

## Telepen

Restore Factory Defaults

Restore the Factory Defaults of Telepen (default)

Enable/Disable Telepen

| Enable Telepen | Disable Telepen (default) |
| --- | --- |

# Serial Commands

The serial programming commands can be used in place of the programming bar codes. Both the serial commands and the programming bar codes will program the scanner. For complete descriptions and examples of each serial programming command, refer to the corresponding programming bar code in this manual.

The device must be set to an TTL-RS232 interface. The following commands can be sent via a PC COM port using terminal emulation software.

## Function Commands

The serial function commands have the following formation:

STX + CMD(see following table) + EXT

| Function | Command | Formation |
| --- | --- | --- |
| Trigger | \xF4 | \x02\xF4\x03 |
| Untrigger | \xF5 | \x02\xF5\x03 |

## Menu Commands

The menu commands have the following syntax:

\x02\xF0\x03 + Menu Number + Setting Value + Character

Character consists of three characters. The details refer to following table.

| Character | Function |
| --- | --- |
| . | Store in ROM |
| ! | Store in RAM |
| ? | Query the current setting |
| * | Return the valid setting range |

Example:

| Action | Send Commands | Response |
| --- | --- | --- |
| Set the unit default | \x02\xF0\x030D0100. | 0D0100 [ACK]. |
| Set unit mid-level illumination 1 (saved in ROM) | \x02\xF0\x038002001. | 8002001[ACK]. |
| Set unit mid-level illumination 1 (saved in RAM) | \x02\xF0\x038002001! | 8002001[ACK]! |
| Query the current illumination level | \x02\xF0\x03800200?. | 8002000[ACK]. |
| Query the range value of barcode scanning delay | \x02\xF0\x03080B08*. | 080B080-30000[ACK]. |

## Menu number

| Function | Details | Menu Number | Setting Value |
| --- | --- | --- | --- |
| Restore All Factory Defaults | Activate Defaults | 0D0100 | NA |
| Interface | 232 Interface | 091804 | NA |
| 232 Interface Baud rate | 2400 BPS | 060702 | 3 |
|  | 4800 BPS |  | 4 |
|  | 9600 BPS |  | 5 |
|  | 19200 BPS |  | 6 |
|  | 38400 BPS |  | 7 |
|  | 57600 BPS |  | 8 |
|  | 115200 BPS(default) |  | 9 |
| 232 Interface Data Bit & Parity Check& Stop Bit | None Parity /8 Data Bits/1 Stop Bit(default) | 060703 | 2 |
|  | Even Parity /8 Data Bits/1 Stop Bit |  | 5 |
|  | Odd Parity /8 Data Bits/1 Stop Bit |  | 8 |
| USB HID-KB | USB HID-KB | 091809 | NA |
|  | No Case Conversion (default) | 060D02 | 0 |
|  | Convert All to Upper Case |  | 1 |
|  | Convert All to Lower Case |  | 2 |
| USB COM Port | USB COM Port Emulation | 060500 | 130 |
| Scanning Mode | Trigger Mode(default) | 091A00 | NA |
|  | Presentation Mode | 090901 | NA |
|  | Trigger Mode with cell phone reading | 091B00 | NA |
|  | Presentation Mode with cell phone reading | 090902 | NA |
| Inverse Color | Off(default) | 024B00 | 0 |
|  | Only Inverse On |  | 1 |
|  | Inverse and Normal Both On |  | 2 |
| Illumination for Trigger Mode | High Level Illumination (default) | 040102 | 0 |
|  | Mid Level Illumination 1 |  | 1 |
|  | Mid Level Illumination 2 |  | 2 |
|  | Low Level Illumination |  | 3 |
|  | Illumination OFF |  | 4 |
| Illumination for Presentation Mode in Scanning | High Level Illumination (default) | 040100 | 0 |
|  | Mid Level Illumination 1 |  | 1 |
|  | Mid Level Illumination 2 |  | 2 |
|  | Low Level Illumination |  | 3 |
|  | Illumination OFF |  | 4 |
| Illumination for Presentation Mode in Idle | High Level Illumination | 040101 | 0 |
|  | Mid Level Illumination 1 |  | 1 |
|  | Mid Level Illumination 2 |  | 2 |
|  | Low Level Illumination (default) |  | 3 |
|  | Illumination OFF |  | 4 |
| Beeper - Good Read | On (default) | 050210 | 1 |
|  | Off |  | 0 |
| Beeper Tone – Good Read | Low | 05021D | 1 |
|  | Middle |  | 2 |
|  | High (default) |  | 3 |
| Beeper Duration – Good Read | Normal (default) | 050216 | 0 |
|  | Short |  | 1 |
| Beeper Number-Good Read | 1(default) | 05020E | 1 |
|  | 2 |  | 2 |
|  | 3 |  | 3 |
|  | 4 |  | 4 |
|  | 5 |  | 5 |
| Barcode Scanning Delay | No Delay (default) | 080B08 | 0 |
|  | Delay 500MS |  | 500 |
|  | Delay 2000ms |  | 2000 |
| Add Specified Suffix | Add CR | 080500 | 990D |
|  | Add LF |  | 990A |
|  | Add CRLF |  | 990D0A |
|  | Add TAB |  | 9909 |
| Prefix | Add Prefix | 080400 | 99XX |
| Suffix | Add Suffix | 080500 | 99XX |
| Clear all prefix and suffix (default) | Clear all prefix and suffix (default) | 080404;080507 | NA |
| Save | Save setting | 0D0400 | NA |
| Not Save | Not save setting | 0D0500 | NA |
| Enable/Disable All Symbologies | Enable All Symbologies | 020100 | 1 |
|  | Disable All Symbologies |  | 0 |
| Code 128 | Restore the Factory Defaults of Code 128 (default) | 020A00 | NA |
|  | Enable Code 128 (default) | 020A01 | 1 |
|  | Disable Code 128 |  | 0 |
|  | Code 128 max length | 020A03 | Min~90 |
|  | Code 128 min length | 020A02 | 0~max |
| EAN-8 | Restore the Factory Defaults of EAN-8 (default) | 021400 | NA |
|  | Enable EAN-8 (default) | 021401 | 1 |
|  | Disable EAN-8 |  | 0 |
|  | Transmit EAN-8 Check Digit (default) | 021402 | 1 |
|  | Do Not Transmit EAN-8 Check Digit |  | 0 |
|  | Enable 2-Digit Add-On Code | 021403 | 1 |
|  | Disable 2-Digit Add-On Code (default) |  | 0 |
|  | Enable 5-Digit Add-On Code | 021404 | 1 |
|  | Disable 5-Digit Add-On Code (default) |  | 0 |
|  | EAN-8 Add-On Code Required | 021405 | 1 |
|  | EAN-8 Add-On Code Not Required (default) |  | 0 |
|  | Enable ENA/JAN-8 Addenda Separator (default) | 021406 | 1 |
|  | Disable ENA/JAN-8 Addenda Separator UPC |  | 0 |
| EAN-13 | Restore the Factory Defaults of EAN-13 (default) | 021300 | NA |
|  | Enable EAN-13 (default) | 021301 | 1 |
|  | Disable EAN-13 |  | 0 |
|  | Transmit EAN-13 Check Digit (default) | 021302 | 1 |
|  | Do Not Transmit EAN-13 Check Digit |  | 0 |
|  | Enable 2-Digit Add-On Code | 021303 | 1 |
|  | Disable 2-Digit Add-On Code(default) |  | 0 |
|  | Enable 5-Digit Add-On Code | 021304 | 1 |
|  | Disable 5-Digit Add-On Code(default) |  | 0 |
|  | EAN-13 Add-On Code Required | 021305 | 1 |
|  | EAN-13 Add-On Code Not Required (default) |  | 0 |
|  | Enable ENA/JAN-13 Addenda Separator (default) | 021306 | 1 |
|  | Disable ENA/JAN-13 Addenda Separator |  | 0 |
|  | Enable ISBN Translate | 021307 | 1 |
|  | Disable ISBN Translate (default) |  | 0 |
| UPC-E | Restore the Factory Defaults of UPC-E (default) | 021200 | NA |
|  | Enable UPC-E0 (default) | 021201 | 1 |
|  | Disable UPC-E0 |  | 0 |
|  | Enable UPC-E1(default) | 021202 | 1 |
|  | Disable UPC-E1 |  | 0 |
|  | Enable UPC-E0 Check Digit (default) | 021204 | 1 |
|  | Disable UPC-E0 Check Digit |  | 0 |
|  | Enable UPC-E0 Expand | 021203 | 1 |
|  | Disable UPC-E0 Expand (default) |  | 0 |
|  | Enable UPC-E0 Required | 021208 | 1 |
|  | Disable UPC-E0 Required (default) |  | 0 |
|  | Enable UPC-E0 Separator (default) | 021209 | 1 |
|  | Disable UPC-E0 Separator |  | 0 |
|  | Enable UPC-E0 Number System (default) | 021205 | 1 |
|  | Disable UPC-E0 Number System |  | 0 |
|  | Enable 2 Digit Addenda | 021206 | 1 |
|  | Disable 2 Digit Addenda (default) |  | 0 |
|  | Enable 5 Digit Addenda | 021207 | 1 |
|  | Disable 5 Digit Addenda (default) |  | 0 |
| UPC-A | Restore the Factory Defaults of UPC-A (default) | 021100 | NA |
|  | Enable UPC-A (default) | 021101 | 1 |
|  | Disable UPC-A |  | 0 |
|  | Enable UPC-A Check Digit (default) | 021102 | 1 |
|  | Disable UPC-A Check Digit |  | 0 |
|  | Enable UPC-A Required | 021106 | 1 |
|  | Disable UPC-A Required (default) |  | 0 |
|  | Enable UPC-A Separator (default) | 021107 | 1 |
|  | Disable UPC-A Separator |  | 0 |
|  | Enable UPC-A Number System (default) | 021103 | 1 |
|  | Disable UPC-A Number System |  | 0 |
|  | Enable 2 Digit Addenda | 021104 | 1 |
|  | Disable 2 Digit Addenda (default) |  | 0 |
|  | Enable 5 Digit Addenda | 021105 | 1 |
|  | Disable 5 Digit (default) |  | 0 |
| Interleaved 2 of 5 | Restore the Factory Defaults of Interleaved 2 of 5 (default) | 020400 | NA |
|  | Enable Industrial 2 of 5 | 020401 | 1 |
|  | Disable Interleaved 2 of 5 |  | 0 |
|  | No check Char (default) | 020402 | 0 |
|  | Validate and Transmit |  | 2 |
|  | Validate not Transmit |  | 1 |
|  | Interleaved 2 of 5 max length | 020404 | Min~80 |
|  | Interleaved 2 of 5 min length | 020403 | 2~max |
| Matrix 2 of 5 | Restore the Factory Defaults of Matrix 2 of 5 (default) | 020800 | NA |
|  | Enable Matrix 2 of 5 | 020801 | 1 |
|  | Disable Matrix 2 of 5 (default) |  | 0 |
|  | Matrix 2 of 5 max length | 020803 | Min~80 |
|  | Matrix 2 of 5 min length | 020802 | 1~max |
| Industrial 2 of 5 | Restore the Factory Defaults of Industrial 2 of 5 (default) | 020600 | NA |
|  | Enable Industrial 2 of 5 | 020601 | 1 |
|  | Disable Industrial 2 of 5 (default) |  | 0 |
|  | Industrial 2 of 5 max length | 020603 | Min~48 |
|  | Industrial 2 of 5 min length | 020602 | 1~max |
| Code 39 | Restore the Factory Defaults of Code 39 (default) | 020300 | NA |
|  | Enable Code 39 (default) | 020301 | 1 |
|  | Disable Code 39 |  | 0 |
|  | Transmit Start/Stop Character | 020305 | 1 |
|  | Do not Transmit Start/Stop Character (default) |  | 0 |
|  | No Check Char (default) | 020304 | 0 |
|  | Validate and Transmit |  | 2 |
|  | Validate no Transmit |  | 1 |
|  | Enable Code 39 Full ASCII | 020302 | 1 |
|  | Disable Code 39 Full ASCII (default) |  | 0 |
|  | Code 39 max length | 020308 | Min~48 |
|  | Code 39 min length | 020307 | 0~max |
| Coda bar | Restore the Factory Defaults of Coda bar (default) | 020200 | NA |
|  | Enable Coda bar (default) | 020201 | 1 |
|  | Disable Coda bar |  | 0 |
|  | Coda bar max length | 020206 | Min~60 |
|  | Coda bar min length | 020205 | 2~max |
|  | Transmit Start/Stop Character | 020202 | 1 |
|  | Do not Transmit Start/Stop Character (default) |  | 0 |
|  | No Check Char (default) | 020203 | 0 |
|  | Validate and Transmit |  | 2 |
|  | Validate no Transmit |  | 1 |
| Code 93 | Restore the Factory Defaults of Code 93 (default) | 020D00 | NA |
|  | Enable Code 93 | 020D01 | 1 |
|  | Disable Code 93 (default) |  | 0 |
|  | Code 93 max length | 020D03 | Min~80 |
|  | Code 93 min length | 020D02 | 0~max |
| GS1-128 | Restore the Factory Defaults of GS1-128 (default) | 020B01 | NA |
|  | Enable GS1-128 (default) | 020B00 | 1 |
|  | Disable GS1-128 |  | 0 |
|  | GS1-128 max length | 020B03 | Min~80 |
|  | GS1-128 min length | 020B02 | 0~max |
| MSI | Restore the Factory Defaults of MSI (default) | 020E00 | NA |
|  | Enable MSI | 020E01 | 1 |
|  | Disable MSI (default) |  | 0 |
|  | MSI max length | 020E04 | Min~48 |
|  | MSI min length | 020E03 | 4~max |
|  | Validate Type 10, No Transmit (Default) | 020E02 | 0 |
|  | Validate Type 10, Transmit |  | 1 |
|  | Validate 2 Type 10 Characters, No Transmit |  | 2 |
|  | Validate 2 Type 10 Characters, Transmit |  | 3 |
|  | Validate Type 11 Then Type 10 Character, No Transmit |  | 4 |
|  | Validate Type 11 Then Type 10 Characters, Transmit |  | 5 |
|  | Disable MSI Check Characters |  | 6 |
| PDF 417 | Restore the Factory Defaults of PDF 417 (default) | 021F00 | NA |
|  | Enable PDF 417 (default) | 021F01 | 1 |
|  | Disable PDF 417 |  | 0 |
|  | PDF417 max length | 021F06 | Min~2750 |
|  | PDF417 min length | 021F05 | 1~max |
| Micro PDF 417 | Restore the Factory Defaults of Micro PDF 417 (default) | 022000 | NA |
|  | Enable Micro PDF 417 | 022001 | 1 |
|  | Disable Micro PDF 417 (default) |  | 0 |
|  | Micro PDF417 max length | 022003 | Min~2750 |
|  | Micro PDF417 min length | 022002 | 1~max |
| QR Code | Restore the Factory Defaults of QR Code (default) | 023700 | NA |
|  | Enable QR Code (default) | 023701 | 1 |
|  | Disable QR Code |  | 0 |
|  | QR max length | 023703 | Min~7089 |
|  | QR max length | 023702 | 1~max |
| Data Matrix | Restore the Factory Defaults of Data Matrix (default) | 023600 | NA |
|  | Enable Data Matrix (default) | 023601 | 1 |
|  | Disable Data Matrix |  | 0 |
|  | Data Matrix max length | 023603 | Min~3116 |
|  | Data Matrix min length | 023602 | 1~max |
| Maxi code | Restore the Factory Defaults of Maxi code (default) | 023400 | NA |
|  | Enable Maxi code | 023401 | 1 |
|  | Disable Maxi code (default) |  | 0 |
|  | Maxi Code max length | 023403 | Min~150 |
|  | Maxi Code min length | 023402 | 1~max |
| Aztec | Restore the Factory Defaults of Aztec (default) | 023300 | NA |
|  | Enable Aztec | 023301 | 1 |
|  | Disable Aztec (default) |  | 0 |
|  | Aztec max length | 023306 | Min~3832 |
|  | Aztec min length | 023305 | 0~Max |
| Hanxin | Restore the Factory Defaults of Hanxin (default) | 023800 | NA |
|  | Enable Hanxin | 023801 | 1 |
|  | Disable Hanxin (default) |  | 0 |
|  | Hanxin max length | 023803 | Min~7833 |
|  | Hanxin min length | 023802 | 1~max |
| China Postal Code | Restore the Factory Defaults of China Postal Code (default) | 021800 | NA |
|  | Enable China Postal Code | 021801 | 1 |
|  | Disable China Postal Code (default) |  | 0 |
| Telepen | Restore the Factory Defaults of Telepen (default) | 021000 | NA |
|  | Enable Telepen | 021001 | 1 |
|  | Disable Telepen (default) |  | 0 |

# Appendix

## Appendix 1: AIM ID Table

| Symbology | AIM ID | Remark |
| --- | --- | --- |
| EAN-13 | ]E0 | Standard EAN-13 |
|  | ]E3 | EAN-13 + 2/5-Digit Add-On Code |
| EAN-8 | ]E4 | Standard EAN-8 |
|  | ]E4…]E1… | EAN-8 + 2-Digit Add-On Code |
|  | ]E4…]E2… | EAN-8 + 5-Digit Add-On Code |
| UPC-E | ]E0 | Standard UPC-E |
|  | ]E3 | UPC-E + 2/5-Digit Add-On Code |
| UPC-A | ]E0 | Standard UPC-A |
|  | ]E3 | UPC-A + 2/5-Digit Add-On Code |
| Code 128 | ]C0 | Standard Code 128 |
| GS1-128 (UCC/EAN-128) | ]C1 | FNC1 is the character right after the start character |
| AIM-128 | ]C2 | FNC1 is the 2nd character after the start character |
| ISBT-128 | ]C4 |  |
| Interleaved 2 of 5 | ]I0 | No parity check |
|  | ]I1 | Transmit check digit after parity check |
|  | ]I3 | Do not transmit check digit after parity check |
| ITF-6 | ]I1 | Transmit check digit |
|  | ]I3 | Do not transmit check digit |
| ITF-14 | ]I1 | Transmit check digit |
|  | ]I3 | Do not transmit check digit |
| Industrial 2 of 5 | ]S0 | Not specified |
| Standard 2 of 5 | ]R0 | No parity check |
|  | ]R8 | One check digit, MOD10; do not transmit check digit |
|  | ]R9 | One check digit, MOD10; transmit check digit |
| Code 39 | ]A0 | Transmit barcodes as is; Full ASCII disabled; no parity check |
|  | ]A1 | One check digit, MOD43; transmit check digit |
|  | ]A3 | One check digit, MOD43; do not transmit check digit |
|  | ]A4 | Full ASCII enabled; no parity check |
|  | ]A5 | Full ASCII enabled; transmit check digit |
|  | ]A7 | Full ASCII enabled; do not transmit check digit |
| Codabar | ]F0 | Standard Codabar |
|  | ]F2 | Transmit check digit after parity check |
|  | ]F4 | Do not transmit check digit after parity check |
| Symbology | AIM ID | Remark |
| Code 93 | ]G0 | Standard Code 93 |
| Code 11 | ]H0 | One check digit MOD11; transmit check digit |
|  | ]H1 | Two check digits, MOD11/MOD11; transmit check digit |
|  | ]H3 | Do not transmit check digit after parity check |
|  | ]H9 | No parity check |
| GS1-DataBar (RSS) | ]e0 | Standard GS1-DataBar |
| Plessey | ]P0 | Standard Plessey |
| MSI-Plessey | ]M0 | One check digit, MOD10; transmit check digit |
|  | ]M1 | One check digit, MOD10; do not transmit check digit |
|  | ]M8 | Two check digits |
|  | ]M9 | No parity check |
| Matrix 2 of 5 | ]X0 | Specified by the manufacturer |
|  | ]X1 | No parity check |
|  | ]X2 | One check digit, MOD10; transmit check digit |
|  | ]X3 | One check digit, MOD11; do not transmit check digit |
| ISBN | ]X4 | Standard ISBN |
| ISSN | ]X5 | Standard ISSN |
| PDF417 | ]L0 | Comply with 1994 PDF417 specifications |
| Data Matrix | ]d0 | ECC000 - ECC140 |
|  | ]d1 | ECC200 |
|  | ]d2 | ECC200, FNC1 is the 1st or 5th character after the start character |
|  | ]d3 | ECC200, FNC1 is the 2nd or 6th character after the start character |
|  | ]d4 | ECC200, ECI included |
|  | ]d5 | ECC200, FNC1 is the 1st or 5th character after the start character,ECI included |
|  | ]d6 | ECC200, FNC1 is the 2nd or 6th character after the start character,ECI included |
| QR Code | ]Q0 | QR1 |
|  | ]Q1 | 2005 version, ECI excluded |
|  | ]Q2 | 2005 version, ECI included |
|  | ]Q3 | QR Code 2005, ECI excluded, FNC1 is the 1st character after the start character |
|  | ]Q4 | QR Code 2005, ECI included, FNC1 is the 1st character after the start character |
|  | ]Q5 | QR Code 2005,ECI excluded,FNC1 is the 2nd character after the start character |
|  | ]Q6 | QR Code 2005, ECI included, FNC1 is the 2nd character after the start character |

Reference: ISO/IEC 15424:2008 Information technology – Automatic identification and data capture techniques – Data Carrier

Identifiers (including Symbology Identifiers).

## Appendix 2: ASCII Table

| Hex |  | Dec |  | Char |
| --- | --- | --- | --- | --- |
| 00 |  | 0 |  | NUL (Null char.) |
| 01 |  | 1 |  | SOH (Start of Header) |
| 02 |  | 2 |  | STX (Start of Text) |
| 03 |  | 3 |  | ETX (End of Text) |
| 04 |  | 4 |  | EOT (End of Transmission) |
| 05 |  | 5 |  | ENQ (Enquiry) |
| 06 |  | 6 |  | ACK (Acknowledgment) |
| 07 |  | 7 |  | BEL (Bell) |
| 08 |  | 8 |  | BS (Backspace) |
| 09 |  | 9 |  | HT (Horizontal Tab) |
| 0a |  | 10 |  | LF (Line Feed) |
| 0b |  | 11 |  | VT (Vertical Tab) |
| 0c |  | 12 |  | FF (Form Feed) |
| 0d |  | 13 |  | CR (Carriage Return) |
| 0e |  | 14 |  | SO (Shift Out) |
| 0f |  | 15 |  | SI (Shift In) |
| 10 |  | 16 |  | DLE (Data Link Escape) |
| 11 |  | 17 |  | DC1 (XON) (Device Control 1) |
| 12 |  | 18 |  | DC2 (Device Control 2) |
| 13 |  | 19 |  | DC3 (XOFF) (Device Control 3) |
| 14 |  | 20 |  | DC4 (Device Control 4) |
| 15 |  | 21 |  | NAK (Negative Acknowledgment) |
| 16 |  | 22 |  | SYN (Synchronous Idle) |
| 17 |  | 23 |  | ETB (End of Trans. Block) |
| 18 |  | 24 |  | CAN (Cancel) |
| 19 |  | 25 |  | EM (End of Medium) |
| 1a |  | 26 |  | SUB (Substitute) |
| 1b |  | 27 |  | ESC (Escape) |
| 1c |  | 28 |  | FS (File Separator) |
| 1d |  | 29 |  | GS (Group Separator) |
| 1e | 30 |  | RS (Request to Send) |  |
| 1f | 31 |  | US (Unit Separator) |  |
| Hex |  | Dec |  | Char |
| 20 | 32 |  | SP (Space) |  |
| 21 | 33 |  | ! (Exclamation Mark) |  |
| 22 | 34 |  | " (Double Quote) |  |
| 23 | 35 |  | # (Number Sign) |  |
| 24 | 36 |  | $ (Dollar Sign) |  |
| 25 | 37 |  | % (Percent) |  |
| 26 | 38 |  | & (Ampersand) |  |
| 27 | 39 |  | ` (Single Quote) |  |
| 28 | 40 |  | ( (Right / Closing Parenthesis) |  |
| 29 | 41 |  | ) (Right / Closing Parenthesis) |  |
| 2a | 42 |  | * (Asterisk) |  |
| 2b | 43 |  | + (Plus) |  |
| 2c | 44 |  | , (Comma) |  |
| 2d | 45 |  | - (Minus / Dash) |  |
| 2e | 46 |  | . (Dot) |  |
| 2f | 47 |  | / (Forward Slash) |  |
| 30 | 48 |  | 0 |  |
| 31 | 49 |  | 1 |  |
| 32 | 50 |  | 2 |  |
| 33 | 51 |  | 3 |  |
| 34 | 52 |  | 4 |  |
| 35 | 53 |  | 5 |  |
| 36 | 54 |  | 6 |  |
| 37 | 55 |  | 7 |  |
| 38 | 56 |  | 8 |  |
| 39 | 57 |  | 9 |  |
| 3a | 58 |  | : (Colon) |  |
| 3b | 59 |  | ; (Semi-colon) |  |
| 3c | 60 |  | < (Less Than) |  |
| 3d | 61 |  | = (Equal Sign) |  |
| 3e |  | 62 |  | > (Greater Than) |
| 3f |  | 63 |  | ? (Question Mark) |
| Hex |  | Dec |  | Char |
| 40 |  | 64 |  | @ (AT Symbol) |
| 41 |  | 65 |  | A |
| 42 |  | 66 |  | B |
| 43 |  | 67 |  | C |
| 44 |  | 68 |  | D |
| 45 |  | 69 |  | E |
| 46 |  | 70 |  | F |
| 47 |  | 71 |  | G |
| 48 |  | 72 |  | H |
| 49 |  | 73 |  | I |
| 4a |  | 74 |  | J |
| 4b |  | 75 |  | K |
| 4c |  | 76 |  | L |
| 4d |  | 77 |  | M |
| 4e |  | 78 |  | N |
| 4f |  | 79 |  | O |
| 50 |  | 80 |  | P |
| 51 |  | 81 |  | Q |
| 52 |  | 82 |  | R |
| 53 |  | 83 |  | S |
| 54 |  | 84 |  | T |
| 55 |  | 85 |  | U |
| 56 |  | 86 |  | V |
| 57 |  | 87 |  | W |
| 58 |  | 88 |  | X |
| 59 |  | 89 |  | Y |
| 5a |  | 90 |  | Z |
| 5b |  | 91 |  | [ (Left / Opening Bracket) |
| 5c |  | 92 |  | \ (Back Slash) |
| 5d |  | 93 |  | ] (Right / Closing Bracket) |
| 5e | 94 |  | ^ (Caret / Circumﬂex) |  |
| 5f | 95 |  | _ (Underscore) |  |
| Hex |  | Dec |  | Char |
| 60 | 96 |  | ' (Grave Accent) |  |
| 61 | 97 |  | a |  |
| 62 | 98 |  | b |  |
| 63 | 99 |  | c |  |
| 64 | 100 |  | d |  |
| 65 | 101 |  | e |  |
| 66 | 102 |  | f |  |
| 67 | 103 |  | g |  |
| 68 | 104 |  | h |  |
| 69 | 105 |  | i |  |
| 6a | 106 |  | j |  |
| 6b | 107 |  | k |  |
| 6c | 108 |  | l |  |
| 6d | 109 |  | m |  |
| 6e | 110 |  | n |  |
| 6f | 111 |  | o |  |
| 70 | 112 |  | p |  |
| 71 | 113 |  | q |  |
| 72 | 114 |  | r |  |
| 73 | 115 |  | s |  |
| 74 | 116 |  | t |  |
| 75 | 117 |  | u |  |
| 76 | 118 |  | v |  |
| 77 | 119 |  | w |  |
| 78 | 120 |  | x |  |
| 79 | 121 |  | y |  |
| 7a | 122 |  | z |  |
| 7b | 123 |  | { (Left/ Opening Brace) |  |
| 7c | 124 |  | ¦ (Vertical Bar) |  |
| 7d | 125 |  | } (Right/Closing Brace) |  |
| 7e | 126 |  | ~ (Tilde) |  |
| 7f | 127 |  | DEL (Delete) |  |

## Appendix 3: Digit Barcodes

| 0 | 1 | 2 | 3 |
| --- | --- | --- | --- |
| 4 | 5 | 6 | 7 |
| 8 | 9 | A | B |
| C | D | E | F |